D2004

Features: Mini design for homes, small retail stores, offices MJPEG compression High resolution picture quality Record modes: motion detection / time schedule Motion detection: area selection & sensitivity adjustment for each camera PC playback & backup via built-in USB 2.0 port With auto switch display function System self-recovery after power reconnected Password protection IR remote control included Specifications: Model; D2004. Compression; MJPEG (12-20kbytes / frame). Video I/O; BNC 4-channel inputs / 1-channel output. Display Resolution; NTSC: 720x480 PAL: 720x576. Max. Display Frame; NTSC: 120 frames (4x30 frames) PAL: 100 frames (4x25 frames). Max. Record Frame; (Quad Mode) NTSC: 30 frames PAL: 25 frames. (Each Mode) NTSC: 7.5 frames PAL: 6.25 frames. HDD Interface; PATA-100, removable HDD tray, supports to install 250G HDD. Estimated Recording Time; NTSC: 162 hrs. with 250G HDD @ 30fps@ normal picture quality. PAL: 194 hrs. with 250G HDD @ 25fps @ normal picture quality. Record Modes; Schedule, Motion Detection. Record Picture Quality; Low, Normal, High. Display Modes; Each, Quad, Auto-switch. USB Interface; USB2.0. Remote Control; IR remote control. Power Supply; DC 12V / 3A (adapter included). Dimensions (DxWxH); 250x235x38mm.